Mechanical sink or basin mixer

Tall single hole, single control mixer with ceramic cartridge

Hygiene control lever L. 215mm

Reference: 2871T3

Specification description

Deck-mounted tall mechanical sink mixer.

Single hole, single control mixer with high, swivelling spout H. 320mm.

Mechanical mixer supplied with 1 disposable, removable spout.

Ø 35mm classic ceramic cartridge with pre-set maximum temperature limiter.

Body and spout with smooth interiors (reduced bacterial development).

No risk of cross flow between the hot and cold water.

No non-return valves on the inlets.

Thermal shocks are easy to undertake without removing the control lever or shutting off the cold water supply.

Flow rate limited to 9 lpm at 3 bar.

Hygiene control lever L. 185mm requires no manual contact.

Chrome-plated brass body and base.

F3/8" PEX flexibles.

Fixing reinforced via 2 stainless steel rods.

Mechanical mixer ideal for healthcare facilities, retirement and care homes, hospitals and clinics.

Single control mixer suitable for people with reduced mobility.

Mixer with 30-year warranty.