Mechanical mixer for basins

Single hole, single control tall mixer with ceramic cartridge

Mechanical mixer with fixed spout H.160 L.140

Reference: 2565T1

Specification description

Deck-mounted mechanical basin mixer.

Single hole, single control mixer with fixed spout H. 165mm L. 140mm.

Tall mixer with BIOSAFE hygienic outlet suitable for fitting a BIOFIL point-of-use filter.

Ø 35mm classic ceramic cartridge with pre-set maximum temperature limiter.

Body and spout with smooth interiors (reduces bacterial development).

No risk of cross flow between the hot and cold water.

No non-return on the inlets.

Thermal shocks are easy to undertake without removing the control lever or shutting off the cold water supply.

Flow rate regulated at 5 lpm.

Chrome-plated brass body and base.

Hygiene control lever L. 185mm requires no manual contact.

F3/8" PEX flexibles.

Reinforced fixings via 2 stainless steel rods.

Mechanical mixer ideal for healthcare facilities, retirement and care homes, hospitals and clinics.

Single hole mixer suitable for people with reduced mobility.

Mixer with 30-year warranty.