Surgical scrub up trough with high splashback

Polished satin 304 stainless steel - 1 service

With 1 tap holes Ø 22mm per service

Reference: 181100

Specification description

Wall-mounted surgical scrub up trough with splashback 360mm.

Suitable for hospitals and healthcare facilities.

Bacteriostatic 304 stainless steel. Polished satin finish. Stainless steel thickness: 1.2mm.

L. 700mm, 1 service. With 1 tap hole Ø 22mm.

Smooth surface and rounded inside edges for easy cleaning.

Drainage on the right. Without overflow. Supplied with fixing elements.

CE marked. Complies with norm EN 14296.

Weight: 13.8kg.

30-year warranty.