TEMPOSTOP time flow urinal valve - lead-free version

7-second time flow, lead-free version

Recessed, polished satin ss plate, F1/2"

Reference: 779427LF

Specification description

Recessed time flow urinal valve:

For standard or siphon action urinals.

Cross-wall installation.

Adjustable to recess depths from 25 to 45 mm.

Stainless-steel plate 160 x 160 mm.

Compatible with rain water.

Time flow ~7 seconds.

Flow rate pre-set at 0,25 l/sec at 3 bar, adjustable.

Lead-free brass body F1/2".

30-year warranty.

This sanitary ware model is not supplied with recessed waterproof housing, the installer must ensure the impermeability of recessed housing as well as draining any potential leaks and condensation (see installation guide).