Mirror cabinet with 3 functions

Mirror, hand dryer and electronic soap dispenser

No strip - Suitable for people w/ reduced mobility

Reference: 510213

Specification description

Mirror cabinet with 3 functions: mirror, electronic soap dispenser and electrical hand dryer.

304 stainless steel designer cabinet without strip with a matte black melamine finish.

Touch-free for improved hygiene: soap dispenser and hand dryer are activated automatically when hands are detected by infrared sensor.

Quick and easy to install:

- A single electrical connection for all the elements: mains supply with 230V / 50-60Hz.

- Concealed fixings. Mirror cabinet supplied with stainless steel screws for concrete or reinforced wood.

- Adjustable hinges.

Unit with 3 functions:

4mm thick glass mirror and black glass strip with illuminated pictograms.

Automatic soap dispenser:

- No-waste pump dispenser

- Large capacity tank: 5 litres.

- Suitable for vegetable-based liquid soap with a maximum viscosity of 3,000 mPa.s or 3,000cP, or specific foam soap.

Air-pulse hand dryer:

- High speed.

- Pulsed air at 360 km/h. Airflow: 39 l/second.

- Low energy consumption: 1350W.

- Noise level: 70 dBA.

- Class I, IP23.

- 3-year warranty.

Basin lit by the LED strip.

Vandal-resistant model with lock and standard DELABIE key.

Anti-blocking: stops automatically if used continuously.

Internal storage shelf.

Several cabinets can be installed side-by-side to create a mirror wall: linking screws are provided.

Mirror H.1000mm: facilitates use by people with reduced mobility.

Dimensions : 800 x 223 x 1,000mm.

Weight: 39kg.

Mirror cabinet with 30-year warranty.

CE marked.