Mirror cabinet with 4 functions

Mirror, automatic hand dryer, soap dispenser and tap

No strip - Suitable for people w/ reduced mobility

Reference: 510223

Specification description

Mirror cabinet with 4 functions: mirror, automatic soap dispenser and tap and electronic hand dryer.

304 stainless steel designer cabinet with a matte black melanime finish, without strip.

Touch-free for improved hygiene: soap dispenser, tap and hand dryer are activated automatically when hands are detected by infrared sensor.

Quick and easy to install:

- A single electrical connection for all the elements: mains supply with 230V / 50-60 Hz.

- Connects to the water supply from the top or from inside the cabinet.

- Water connection in M3/8".

- Concealed fixings. Mirror cabinet supplied with stainless steel screws for concrete or reinforced wood.

- Adjustable hinges.

Unit with 4 functions:

4mm thick glass mirror with illuminated pictograms.

Automatic soap dispenser:

- No-waste pump dispenser

- Large capacity tank: 5 litres.

- Suitable for vegetable-based liquid soap with a maximum viscosity of 3,000mPa.s or 3,000cP, or specific foam soap.

Electronic basin tap*:

- Reduced stagnation solenoid valve.

- Flow rate pre-set at 3 lpm at 3 bar, can be adjusted from 1.4 to 6 lpm.

- Scale-resistant flow straightener.

- Duty flush (~60 seconds every 24 hours after the last use).

- Stopcock.

- Adjustable sensor (±10°) for auto-detection.

Air-pulse hand dryer:

- High-speed.

- Pulsed air at 360km/h. Airflow: 39 l/second.

- Low energy consumption: 1350W.

- Noise level: 70 dBA.

- Class I, IP23.

- 3-year warranty.

Basin lit by the LED strip.

Vandal-resistant model with standard DELABIE key.

Anti-blocking: stops automatically if used continuously.

Internal storage shelf.

Several cabinets can be installed side-by-side to create a mirror wall: linking screws are provided.

Miror H.1000mm: facilitates use by people with reduced mobility.

Dimensions: 600 x 223 x 1,000mm.

Weight: 29kg.

Mirror cabinet with 30-year warranty.

CE marked.

* We recommend a PREMIX mixer.